CPE301 – SPRING 2019

# MIDTERM 1

Student Name: David Flores

Student #: 2000881655

Student Email: flored6@unlv.nevada.edu

Primary Github address: https://github.com/David-Floress/submission_da.git

Directory: https://github.com/David-Floress/submission_da

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/Midterm, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328p Xplained mini

FTDI Basic

LM35 (Celsius)

ESP01

Block diagram with pins used in the Atmega328P

Wires (no female headers)

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

- /*

- * Created: 4/5/2019 3:02:41 PM

- * Author : David Flores

- */

- #include <avr/io.h>

- #include <avr/interrupt.h>

- #include <util/delay.h>

- #include <stdlib.h>

- #include <stdint.h>

- #define F_CPU 16000000UL

- #define BAUD 115200

- #define FOSC 16000000

- #define THEUBRR FOSC/8/BAUD -1

- volatile uint8_t ADCvalue;

- volatile unsigned char ADCtemp[5];

- volatile unsigned char CWMODE[] = "AT+CWMODE=3\r\n"; //Set to three for the wifi

- volatile unsigned char WIFI[] = "AT+CWJAP=\"EC4234\", \"G2C26B2B89582\"\r\n"; //Connects to Internet

- volatile unsigned char ENABLE[] = "AT+CIPMUX=0\r\n"; //Connect to WiFi

- volatile unsigned char CIPSTART[] = "AT+CIPSTART=\"TCP\",\"184.106.153.149\",80\r\n"; //TCP strip remote IP, 80 is the port

- volatile unsigned char CIPSEND[] = "AT+CIPSEND=45\r\n"; //Length of data

- volatile unsigned char SEND_DATA[] = "GET /update?key=AHDBMTLB6ZCBMG40&field1="; //Writes API key

- volatile unsigned char PAUSE[] = "\r\n\r\n";

- void send_AT (volatile unsigned char c[]); //sends the arrays to this function

- int main( void )

- {

- /* ADC */

- ADMUX = 0; // uses ADC0

- ADMUX |= (1 << REFS0); // use AVcc as the reference

- ADMUX |= (1 << ADLAR); // Right adjusts for 8 bit resolution

- ADCSRA |= (1 << ADATE); // Set ADC Auto Trigger Enable THEUBRR

- ADCSRB = 0; // 0 for free running ode

- ADCSRA |= (1 << ADEN); // Enable ADC

- ADCSRA |= (1 << ADIE); // Enables Interrupts

- ADCSRA |= (1 << ADSC); // Start ADC conversion

- ADCSRA |= (1 << ADPS2) | (1 << ADPS1) | (1 << ADPS0); // 128 prescaler for 16Mhz

- /* UART */

- UBRR0H = ((THEUBRR) >> 8); //Shift right to store upper 8 bits

- UBRR0L = THEUBRR; //Store lower bits

- UCSR0A |= (1 << U2X0); //Double the USART transmission speed

- UCSR0B |= (1 << TXEN0); // Enables USART transmitter

- UCSR0C |= (1 << UCSZ01) | (1 << UCSZ00); // Set frame: 8data, 1 stop, no parity bit

- _delay_ms(1000);

- send_AT(CWMODE);

- _delay_ms(1000);

- send_AT(WIFI);

- _delay_ms(2000);

- send_AT(ENABLE);

- sei();

- while(1)

- {

- _delay_ms(1000);

- send_AT(CIPSTART); //Connect to thingspeak, port 80

- _delay_ms(1000);

- send_AT(CIPSEND);

- _delay_ms(1000);

- send_AT(SEND_DATA);

- send_AT(ADCtemp); //Sends the temperature data

- send_AT(PAUSE); //A break in the data display

- }

- }

- ISR(ADC_vect)

- {

- volatile unsigned int j=0;

- char temp[5];

- ADCvalue = (ADCH << 1);			//Shifts the value left one place

- itoa(ADCvalue, temp, 10);		//Converts integers to string

- while (j<5) {					//This transfers the temprature string from itoa() to ADCtemp

- ADCtemp[j] = temp[j];

- j++;

- }

- }

- void send_AT(volatile unsigned char c[]) {

- volatile unsigned int i=0, j=0;

- j = 0;					//initialize the counter

- while (c[j] != 0x00) { //while not at end of string

- j++;

- }

- while (i<j) {

- while(!(UCSR0A & (1 << UDRE0))); //If UDRE0 is 1, buffer is empty an can be written to

- UDR0 = c[i];

- i++;

- }

- }

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Part 1:

Firmware successfully installed onto ESP8226 using the FTDI Basic

Part 2:

We check to see if the ESP8226 can connect to thingspeak we use ESPlorer to do so.

The board connects to the ESPlorer and then we can connect to the internet

Program Running:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

I am not going to be including the setup of when I flashed. The only difference was that IO0 was grounded for this part. I did not have female to female wires, so I undid the setup and did the one shown in the next picture without taking the picture. It was a big pain doing it without female connections so I will just include the finished screen from the ESP8266 Download with the Addresses.

ATmega to ESP01 including sensor picture.

In the picture I left the 3.3V wire out during the process I was actually holding it down as seen in the video. That’s the only reason why its not connected.

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=91Y0Box9ECA

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT